03/21/01		QA: QA

RE-SURVEY OF NYE COUNTY EARLY WARNING DRILLING PROGRAM (EWDP) PHASE I BOREHOLES

YMP Site Facilities Department/Field Engineering, Survey Section

A Global Positioning System (GPS) survey for the location of 8 Nye County EWDP Phase I Boreholes was performed on

January 18 and 25, 2001.  The M&O procedure for performing this work is NWI-ESF-008Q, Surveying. Revision 1.

The boreholes listed below were previously surveyed in March 1999.  This survey supercedes DTN MO9904ABNCEWDP.000.

Ref. YMP 129/40-47, 50-55, and Input Request No. 00415.R 	Tabulated below are the location values obtained by this survey.

LATITUDE/LONGITUDE DATUM IS NAD-83, ELEVATION NGVD-29: ( in meters )

# BOREHOLE ID	LATITUDE	LONGITUDE	SLAB/GROUND ELEV.	REMARKS

NC-EWDP-01DX	36 42' 33.526"N	116 35' 18.003"W	803.31m/803.18m	Top of .37m  casing @ 804.18m

NC-EWDP-01S	36 42' 33.386"N	116 35' 17.880"W	803.12m/803.12m	Top of .07m  well string @ 803.91m

NC-EWDP-02D	36 39' 38.524"N	116 27' 56.831"W	N/A/801.23m	No slab or casing exists

NC-EWDP-03D	36 40' 53.597"N	116 32' 17.048"W	798.47m/798.33m	Top of .16m  casing  @ 799.16m

NC-EWDP-03S	36 40' 53.613"N	116 32' 17.179"W	798.40m/798.27m	Top of .07m  well string @ 799.15m

NC-EWDP-05S	36 40' 11.529"N	116 22' 37.071"W	839.32m/839.22m	Top of .22m casing @ 839.90m

NC-EWDP-09SX	36 41' 44.567"N	116 33' 46.769"W	797.30m/797.30m	Top of .07  well string @ 798.01m

NC-Wasburn-1X	36 39' 50.772"N	116 25' 26.835"W	823.47m/823.37m	Top of .22m  casing  @ 824.12m

NORTHING/EASTING DATUM IS NAD-27, ELEVATION NGVD-29:  ( in feet )

BOREHOLE ID	NORTHING	EASTING	SLAB/GROUND ELEV.		REMARKS

NC-EWDP-01DX	N 713168.77	E 523221.20	2635.54'/2635.09'	Top of 1.20'  casing @ 2638.36'

NC-EWDP-01S	N 713154.61	E 523231.16	2634.91'/2634.91'	Top of .24'  well string @ 2637.50'

NC-EWDP-02D	N 695524.59	E 559178.51	N/A/2628.70'	No slab or casing exists

NC-EWDP-03D	N 703079.68	E 537968.10	2619.66'/2619.21'	Top of .52'  casing @ 2621.91'

NC-EWDP-03S	N 703081.24	E 537957.48	2619.43'/2618.98'	Top of .23'  well string @ 2621.89'

NC-EWDP-05S	N 698929.01	E 585219.64	2753.68'/2753.35'	Top of .72  casing @ 2755.57'

NC-EWDP-09SX	N 708224.95	E 530654.83	2615.80'/2615.80'	Top of .23  well string @ 2618.15

NC-Washburn-1X	N 696791.50	E 571395.79	2701.69'/2701.36'	Top of .72'  casing @ 2703.81'