|  | NYE COUNTY NUCLEAR WASTE REPOSITORY PROJECT OFFICE ###### TEST PLAN |  |  |
| --- | --- | --- | --- |
| TITLE: Construction of Sonic Corehole NC-EWDP-19PB |  |  | Revision: 0 Date: 11-15-03 Page: 1 of 12 |
| TEST Plan Number: TPN-5.1 |  | SUPERSEDES: New Document |  |
| APPROVAL |  | CONCURRENCE |  |

1.0	INTRODUCTION

This test plan (TPN) provides general instructions for the construction of Nye County Nuclear Waste Repository Project Office (NWRPO) Early Warning Drilling Program (EWDP) sonic corehole NC-EWDP-19PB.  These instructions include the following tasks:

•	Drilling and casing of 350 feet of the unsaturated zone.

•	Continuous coring and geophysical logging of the upper 300 feet of the saturated zone.

•	Geologic logging, sampling, and laboratory tests of textural layers within this core.

•	Archival of the continuous core and representative samples from textural layers.

•	Corehole completion with dual-string piezometers.

This TPN references applicable quality assurance (QA) technical procedures (TPs) that provide more detailed instructions to NWRPO personnel for routine data collection. In addition, this TPN references applicable portions of the NWRPO drilling contract that provide detailed drilling and well completion instructions and responsibilities for the drilling contractor.

This TPN applies solely to corehole NC-EWDP-19PB and includes information usually included in work plans developed for major phases of the EWDP to address the construction of a group of related wells.

2.0	Purpose and Justification

The U.S. Department of Energy (DOE) has identified the saturated alluvium in Fortymile Wash as a potential groundwater flow pathway from Yucca Mountain to groundwater users in Amargosa Valley. Nye County believes that collecting continuous core from the upper portion of the alluvial aquifer is necessary in order to identify potential preferential flow pathways and provide small-scale estimates of flow and transport properties. These data will support and complement the interpretation of flow and transport data obtained on a much larger scale from single and cross-hole tracer tests and high rate/volume aquifer pump tests.

To date, only approximately 40 feet of alluvium core have been collected in Fortymile Wash. Samples were collected using drive-core methods from widely spaced intervals in boreholes NC-EWDP-10P, -22PA, -24P, and 29P.  However, this coring approach has proven to be prohibitively expensive. Other coring methods attempted in the vicinity of Yucca Mountain have proven to be largely unsuccessful; that is, samples have been significantly disturbed and/or recovery has been poor.

To fill the need for continuous core in Fortymile Wash, Nye County will construct a corehole using conventional drilling and sonic coring methods at the Alluvial Testing Complex (ATC), the site of previous single-hole tracer tests and planned cross-hole tracer tests.  Approximately 300 feet of continuous sonic core will be collected from the water table at approximately 350 feet below ground surface (bgs), to a total depth of approximately 650 feet bgs.  The corehole will be completed with dual-string piezometers, with screens at depths corresponding to the upper two screens of other multiple-screen wells constructed previously at the site.  These piezometers will be used as injection and/or monitoring points in tracer tests planned for the ATC.

The proposed sonic coring method has not yet been used in studies associated with Yucca Mountain. However, this approach has been used successfully to obtain continuous core in both coarse- and fine-grained alluvium and other unconsolidated sediments at a number of mines and groundwater recharge sites in the southwestern U.S. Moreover, the cost of sonic core per foot at depths below 350 feet has been shown to be one-fourth to one-tenth the cost of drive core. Nye County believes that the success of sonic coring at other locations, plus its relatively low cost, justify the coring investigation described in the following.

3.0	Scope of Work For Drilling, Coring, Logging, and Well completion

3.1	Responsibilities and Pre- and Post-Drilling Requirements

3.1.1	Responsibilities, Chain of Command, and Communication

The Nye County NWRPO On-Site Geotechnical Representative will be the Principle Investigator (PI) responsible for supervising all technical data collection described in this plan. The NWRPO-designated field representative (NDFR), in most cases the contract managing geologist, will supervise the contract geologist and technicians, collectively referred to as NWRPO field personnel herein. NWRPO field personnel are responsible for conducting the activities described in this TPN.

The drilling contractor is responsible for the drilling, coring, and well construction specified in the NWRPO drilling contract documents (NWRPO, 2003). A drilling contractor-designated field representative (CDFR) will direct all drilling contractor activities, with the exception of the  NWRPO-directed activities specified in the drilling contract, in which the NDFR or designee is responsible for directing the work. The NDFR and CDFR will communicate regularly and review, approve, and sign daily drilling records that document contract billable items.

3.1.2	Site Location, Surveying, and Pad Preparation

NC-EWDP-19PB will be located in lower Fortymile Wash approximately 5 miles northwest of Lathrop Wells, Nevada, within approximately 160 ft north-northeast of existing well NC-EWDP-19D (Figure 1).

When the final location is determined by the NWRPO, a stake will be driven at the proposed wellhead location, the location surveyed using the global positioning system (GPS), and the latitude and longitude recorded in the field notes to the nearest 0.01 second.  The elevation estimate and the tolerance of the position will also be included, if the GPS has the capability to provide those parameters. All original GPS survey data collected will be transmitted to the NWRPO QA Records Center (QARC) along with associated metadata.  Any additional processing of the data will also be transmitted to the QARC.

Since a large (i.e., approximately 300 by 300 feet) graded and fenced drilling pad is present at the site, no additional site preparation work will be required.

3.1.3	Permitting, License, and Reporting Requirements

The NWRPO, as the well owner, has completed all required federal and state permits. These permits were obtained for previous wells drilled at this site.  Since NC-EWDP-19PB is located on an existing, fenced, and previously approved drill site, no additional permits are required from the U.S. Department of the Interior, U.S. Bureau of Land Management (BLM), Nevada Division of Water Resources (NDWR), or Nevada Division of Water Pollution Control.

The drilling contractor will be required to notify the NDWR before drilling operations by submitting a Notice of Intent to Drill at least 3 working days before the rig is set up, per the requirements of Nevada Administrative Code (NAC) 534.320. The Notice of Intent to Drill will list any permits or waivers issued previously by the NDWR; the NWRPO will supply the drilling contractor with this list.

The drilling contractor will provide the NWRPO with copies of each individual driller’s license before drilling operations begin. Per NAC 534.330, each individual driller will carry his license when he is present at the drilling site and produce the license upon request by an NDWR representative. At least one driller with a Nevada license will be present at the site when the drill rig is operating.

The drilling contractor is also required to meet the reporting requirements of Nevada Revised Statute (NRS) 534.170 and NAC 534.340 by submitting a completed Driller’s Report and Record of Work to the NDWR within 30 days of completion for each well, with copies of these documents transmitted to the NWRPO.

    - Mobilization and Demobilization

Mobilization will be considered complete when the following steps have been taken:

1.	The rig and associated equipment have been inspected and approved by the NDFR as being in clean condition and good working order.

2.	A containment mat has been installed under the rig.  The drilling contractor will use an appropriately heavy-gauge, single-piece plastic mat under the rig and other equipment, as appropriate, to contain all leaks of hydraulic oil, lubricants, or other liquids. The mat should be constructed at least 4 inches deep with a border of wooden skids or other material.

3.	The format of the daily drilling record (i.e., field ticket) has been reviewed and approved by the CDFR.

4.	Material safety data sheets (MSDSs) for all applicable materials on site have been submitted to the NDFR.

5.	All proofs of insurance, personnel training, and other certifications as specified in the contract have been submitted to the NWRPO.

6.	All State of Nevada requirements for the drilling contractor  (e.g., Notice of Intent to Drill) have been met, and applicable documents submitted to the NDWR, with copies to the NWRPO.

7.	The rig and its associated equipment (e.g., drilling fluid handling and sampling systems) have been set up on the proposed hole location and are ready to conduct linear footage drilling or other activities as directed by the NDFR.

8.	All personnel, equipment, tools, and material required under the contract are on location, except those not needed immediately. The drilling contractor may use the NWRPO lay-down yard in Lathrop Wells for equipment storage; however, rig time incurred while waiting for such equipment to be retrieved will be at the drilling contractor’s expense.

Demobilization will be considered complete when the following steps have been taken.

1.	Tasks specified in the contract are complete or exempt from completion by approval from the NWRPO.

2.	Any pits and berms on the drill site have been graded to approximately the original elevation.

3.	All personnel, equipment, tools, unused materials, and drilling-related debris have been removed from the drill site, as well as from the NWRPO lay-down yard.

3.2	Drilling, Coring, and Well Completion

The alluvium of concern will include alternating thin (i.e., sometimes less than 2.5-foot-thick) layers of relatively clean sand and gravel, sand and gravel with silt and/or clay, and silty sand with gravel.  A summary lithologic log for a well approximately 120 feet from NC-EWDP-19PB is shown on Figure 2.

3.2.1	Summary of Drilling Contractor Scope of Work

The detailed scope of work for drilling, coring, and well completion is included in the drilling contract and included as Attachment A.  In summary, this scope of work includes the following steps:

1.	Using conventional rotary, dual rotary, or another proven drilling method, drill a borehole and install a 9-inch diameter or larger conductor casing to approximately 350 feet bgs, approximately 10 feet above the water table.

2.	Collect nearly continuous sonic core from approximately 350 to 650 feet bgs using, to the extent possible, a 6-inch-diameter steel core casing (i.e., core barrel) inside an 8-inch-diameter steel drill casing.  Switch to a 3.5-inch core barrel inside a 5.5-inch drill casing when advancement of the larger sonic drilling/coring system is no longer feasible, if jointly determined by the NDFR and CDFR.

3.	Collect 2-foot-long by 4-inch-diameter solid-tube drive-core samples ahead of the larger diameter sonic drilling/coring system at approximately 60-foot intervals beginning at approximately 380 feet bgs.

4.	Install two 2-inch schedule 80 PVC piezometer strings with 20-foot screens and approximately 40-foot sand packs, the first sand pack extending from approximately 400 to 440 feet bgs and the second from approximately 505 to 545 feet bgs.

3.2.2	Well Completion

Figure 3 shows a detailed subsurface completion diagram for the dual-string piezometer with a 9-inch conductor casing grouted into place with concrete grout in a 12¼-inch-diameter borehole.  If the casing is drilled to 350 feet bgs using a dual-rotary method, Figure 3 will be modified to show the casing in contact with the formation below 60 feet and the 60 feet of permanent surface casing in the upper portion of the borehole. Figure 3 may be modified before beginning completion activities to accommodate field conditions.

Figure 4 shows a dual-string piezometer surface completion diagram. If the casing is drilled in place using dual-rotary methods, Figure 4 will be modified to show a larger diameter steel surface casing terminating in the concrete pad.

NC-EWDP-19PB will be completed with the well screen, casing, and stemming materials shown on Figure 3. All stemming materials will be emplaced using tremmie pipe no more than 20 to 30 feet from the bottom of the hole.

To minimize the potential for caving on the dual-string piezometer during the emplacement of stemming materials below 350 feet (i.e., the bottom of the conductor casing), the sonic drill casing should be pulled back in 10- to 30-foot stages. After each pull-back, the bottom of the well will be sounded to determine if caving has occurred. Completion materials will then be emplaced (i.e., tremmied) in each pull-back interval to a depth of approximately 5 feet below the sonic drill casing.

Coarse-grained 8/12 silica sand will be used as sand pack from approximately 10 feet below to 10 feet above each well screen. A two to one by weight sand/granular bentonite mixture will be placed above and below the sand pack to provide a buffer between the sand pack and grout slurry sealants and to provide a solid surface for accurate depth measurements with a measuring tag line. A centrifugal pump will be used to emplace the sand pack as well as the bentonite mixture.

A bentonite grout slurry will then be emplaced to provide a seal in the annular space between the casing and the formation wall. This slurry will be emplaced upward by tremmie pipe and a standard grout pump to displace any fluid present in the interval being grouted. The upper 50 feet of borehole will be filled with a fine-grained dry bentonite by the gravity free-fall method.

3.2.3	Drilling and Coring Specifications

3.2.3.1	Drilling Equipment Requirements

The upper 350 feet of unsaturated alluvium will be drilled by conventional or dual rotary drilling methods. Sonic coring equipment will be used primarily to core and advance the borehole from 350 to 650 feet in saturated alluvium. Equipment specifications for these drilling systems are detailed in the drilling contract (NWRPO, 2003). In addition, drive core will be collected at selected depths in the saturated zone between 350 and 650 feet.  The NWRPO will supply drive core barrels, shoes, and crossover connections for drive coring.

3.2.3.2	Drilling Fluids

Permissible drilling fluids for the upper 350 feet of unsaturated alluvium are limited to water, untreated bentonite mud (e.g., Aqua Gel Gold Seal), and compressed air. The drilling contractor will obtain NWRPO approval before using any other drilling fluid or additive.  No drilling fluids will be permitted when coring saturated alluvium from approximately 350 to 650 feet bgs.

All discharged liquid drilling fluid will be initially collected in a mud pit for onsite storage.  Discharge  rates will be determined by timed volume measurements as appropriate and will be documented in accordance with the requirements of the temporary discharge permit. No water containing drilling additives, batch water, wastewater, cement, or any fluids other than clear water may be discharged offsite. The NWRPO will photograph erosion controls for any offsite discharges.

3.2.3.3	Hole Deviation

The drilling contractor will use methods (e.g., drilling collars) to prevent hole deviation from exceeding 2 degrees from vertical at 350 feet bgs.  In addition, no deviation exceeding 0.5 degrees per 100 feet (i.e., dogleg) is permitted. The drilling contractor will conduct a deviation survey at 350 feet bgs; if there appears to be a significant dogleg, the NWRPO will conduct a survey to determine whether it exceeds the maximum allowed deviation.

3.2.3.4	Nuisance Water

It is anticipated that nuisance water, such as rainfall or surface runoff, may be encountered during well drilling and construction.  The drilling contractor will at all times protect the work from damage by such water and take all due measures to prevent delays in progress of the work caused by such water.  The drilling contractor will dispose of nuisance water without adverse effects onto the adjacent property.

3.2.3.5	Utilities

No utilities will be available at the drilling location. The drilling contractor will provide portable power packs sufficient to meet all drilling and well construction needs. The drilling contractor will purchase all necessary water for drilling operations from well owners.  Construction and makeup water will be fresh water only and the source of the supply will be approved by the NWRPO.  To the extent possible, the NWRPO will facilitate the identification of well owners interested in selling water supplies to the drilling contractor.

3.2.3.6	Depth Control

Depth control will be maintained by the following methods:

•	Direct monitoring: the NWRPO will inventory drill pipe and collars and their sizes before use. During drilling, the NWRPO will document the drilled interval by completing a Drilling/Coring Data Sheet and Tubing and Casing Record as detailed in TP-7.0, Drill Site Management.

•	Depth  sounding: well depths may be periodically sounded with an NWRPO-approved “tag line.”

•	Geophysical logging: logs will be reviewed to compare the total borehole depth and depth of formation tops with the results of monitored and recorded depth controls.

3.2.3.7	Dust Control

Dust will be controlled on the gravel access road by limiting vehicle speed to 25 mph and by spraying water as necessary while digging mud pits.  Compressed air will be injected with water to control dust production (i.e., the air mist method); other drilling fluids used to drill the unsaturated zone do not produce dust. Coring methods required for the saturated zone also do not yield dust.

3.2.4	Other Drilling Contractor Responsibilities

It is the responsibility of the drilling contractor to be aware of, and comply with, the conditions of the EWDP Drilling and Well Construction Health and Safety Plan.  A copy of this plan will be given to the drilling contractor upon the award of the contract and is included in this TPN as Attachment B.

All solid waste, trash, and construction debris will be removed from the site and managed in accordance with applicable regulations. No wastes will be disposed onsite. Hazardous wastes are not expected to be generated during the drilling and monitoring processes; drilling returns are not hazardous wastes.

In compliance with BLM permit requirements, the drilling contractor will take steps to control noxious weeds.  The drilling contractor will steam-clean the undercarriage of all drilling and heavy equipment before entering public lands.

The drilling contractor will excavate one or more shallow pits to manage cuttings and fluids resulting from drilling. Any pits, trenches, or berms constructed during drilling will be filled by the drilling contractor prior to demobilization.  Drill cuttings (i.e., small rock chips and fragments) will be used as fill material.  No borrow materials will be used for fill or grading.  No unsuitable excavated materials are expected to be generated.

After the NWRPO has approved general restoration, Nye County will be responsible for final site reclamation in accordance with BLM requirements.

3.2.5	Groundwater Measurements and Sampling During Drilling

3.2.5.1	First Groundwater Measurements/Sampling

Based on previous data from boreholes and wells completed at this site, first water will be encountered between 350 and 360 feet bgs during sonic coring.  A groundwater level measurement will be made with an electric well sounder, as detailed in TP-9.9, Measurement of Groundwater Levels Using Electric Well Sounders, when the sonic core barrel is removed from the borehole after reaching 360 feet bgs.

No first groundwater samples will be collected or field chemical parameters measured, due to the following:

1.	Groundwater may be impacted from drilling fluids and grout material used while drilling and casing the overlying unsaturated zone.

2.	The groundwater chemistry of the alluvium at this site has likely been impacted from groundwater originating in the underlying volcanic aquifer that has moved upward into the alluvium in response to a significant upward pointing hydraulic gradient via the open multiple-screen boreholes at this site (e.g. NC-EWDP-19D and -19IM2).

3.2.5.2	Saturated Zone Measurements/Sampling

The NWRPO will routinely measure the groundwater level before the start of drilling/coring activities each day. However, groundwater samples will not be collected for the reason stated in item 2 above.

3.2.6 	Geologic Sampling, Logging, and Processing

Unsaturated zone drill cuttings have been collected, logged, and archived from four previous boreholes drilled within 120 feet of the planned borehole.  Therefore, there is no need to collect drill cuttings from the upper 350 feet of unsaturated alluvium in NC-EWDP-19PB.

Nearly continuous sonic core samples will be collected, logged, and processed from 350 to 650 feet bgs following procedures detailed in TP-8.0, Field Collection, Logging, and Processing of Borehole Geologic Samples.  At a few selected depth intervals in saturated alluvium, drive core samples will also be collected, logged, and processed using procedures documented in TP-8.0. Core collection is described briefly in the following.

3.2.6.1	Sonic Core Collection

A single sonic core run will be 10 feet long, where possible; in no cases will a run exceed 10 feet.  In cases where refusal is met before the target 10-foot depth interval is reached, one or more additional core runs will be conducted.

After a core run is brought to the ground surface, the drilling contractor will transfer the core in approximately 2-foot lengths from the core barrel to plastic socks with an inside diameter approximately equal to the outside diameter of the core barrel.

If more than one core run is required to meet the 10-foot target, the corehole will be depth sounded between runs to determine whether core has been left behind in the corehole (i.e., lost core) and/or caving has occurred. Disturbed material will be removed from the hole before the next run.  NWRPO personnel will determine whether disturbed material is lost core or caved material.

After each 10-foot interval has been cored, the drill casing will be advanced to the bottom of the hole.  The core barrel will then be used to clean out disturbed sediments to the bottom of the hole before initiating a new core run. The drilling contractor will remove any fill or caved material from the borehole between runs.

3.2.6.2	Drive Core Collection

Solid-tube drive-core samples will be collected ahead of the larger diameter sonic drilling/coring system at approximately 60-foot intervals, beginning at approximately 380 feet bgs. Disturbed geologic material (i.e., fill) will be removed to the extent reasonably possible from the bottom of the hole before running in the core barrel.  The core barrel will be advanced with the sonic system rather than the downhole air percussion hammer used in most drive core operations. Where possible, core barrels will be advanced a total distance that exceeds the length of the barrel plus the drive shoe by at least several inches to ensure that sediments are tightly packed.

3.2.7	Borehole Geophysical Logging

Two suites of borehole geophysical logs will be conducted in NC-EWDP-19PB by an NWRPO contractor after total depth in reached. A suite of drill string logs (i.e., density, deviation, fluid temperature, natural gamma, moisture, and spectral gamma) will be run inside the sonic drill casings.  After the well is completed, a suite of well completion logs will be run in each of the two piezometer strings. The completion suite will be the same as the drill string suite with the addition of a sonic log and without the spectral gamma log.

In addition, a second NWRPO contractor will run gamma-gamma density and epithermal neutron porosity logs inside the sonic drill casings to total depth. The relatively large nuclear sources on the downhole tools used to produce these logs have the potential to see farther into the formation and be less affected by the sonic drill casings than density and moisture downhole tools.

Both logging companies will be required to meet the calibration, documentation, and deliverable requirements specified in TP-11, Borehole Geophysical Logging Data Identification and Acceptance.

4.0	Core SAMPLE MANAGEMENT AND TESTING

4.1	 Overview

Sonic and drive core sample processing, addressed in TP-8.0, includes collecting representative grab samples from the major textural layers present in sonic core for geologic logging and laboratory testing purposes.

The total numbers of drive and sonic core segments and grab samples of textural layers are summarized in Table 1. The distribution of these samples among different users is shown in Table 2. Distribution will be controlled and documented with the NWRPO Transfer of Custody Form.  Geologic samples will be maintained under chain of custody at all times, either in view of the current holder or secured in locked storage.

All core and grab sample data will be reviewed by NWRPO personnel not directly involved in recording the data and submitted to the QARC with all supporting documentation and metadata.

The NFDR will use the numbers and types of segments in Table 1 to ensure that all necessary containers and packing, marking, and preservation materials are available at the drill site.

4.2	Drive-Core Sample Management

Each 2-foot drive-core barrel will contain three 6-inch and two 3-inch brass liners, as shown on Figure 5. Liners are referred to as segments after they are filled with geologic material. The top and bottom surface of each segment will be geologically logged, as well as the unconsolidated geologic sample from the shoe of the core barrel.

Two of the 6-inch segments from each of the five planned core runs will be sent to the NWRPO testing laboratory for the analyses listed in Table 3. The total number of each test type is also included in Table 3. The laboratory will use the testing methods listed in Table 4. The remaining segments and shoe sample will be assigned to users listed in Table 2 and transported to the DOE Sample Management Facility (SMF) for storage until used.

4.3	Sonic Core and Grab Sample Management

Sonic core will be collected in approximately 2-foot-long plastic socks, referred to as segments in Table 1, and grab samples will be taken of the major textural layers observed in these segments. These textural layers will often extend across adjacent segments and may range in length from less than 1 foot to more than 10 feet. For Table 1, an average length of 2 feet is assumed.

Grab samples will be split into three representative subsamples for NWRPO use (Table 2). One of the subsamples will be used for detailed geologic logging, one for laboratory testing, and one will be stored at the SMF for future use. Laboratory tests for grab subsamples are specified in Table 3.

After digital photographs have been taken of each sonic core segment, the DOE will assume custody of the core segments and transport and store them at the SMF for future use.

5.0	MANAGEMENT

All NWRPO field personnel performing the tasks described in this TPN will be trained in the procedures specifically applicable to the equipment and methods used before conducting work. Personnel will document that they have read and understand this TPN and other applicable QA documents, (i.e., the most recent version TP-7.0, TP-8.0, TP-9.9, and TP-11).

The Quality Assurance Officer is responsible for ensuring that this plan meets QA requirements and that NWRPO field personnel are trained to and comply with the requirements of this TPN. The PI is responsible for the preparation, technical review, and revision of this TPN, as well as oversight of its performance. NWRPO field personnel are responsible for conducting field sampling and testing.

7.0	REFERENCES

NWRPO (Nuclear Waste Repository Project Office), 2002. EWDP Health and Safety Plan, Nye County Department of Natural Resources and Federal Facilities, Pahrump, Nevada.

NWRPO (Nuclear Waste Repository Project Office), 2003. Drilling, Sonic Coring, and Construction of One EWDP Phase V Monitoring Well - Bid Specifications and Request for Bids.  Notice of Invitation to Bid # 2003-27: Drilling, Sonic Coring, and Construction of One EWDP Phase V Monitoring Well.  Nye County Department of Natural Resources and Federal Facilities. Pahrump, Nevada.

NAC (Nevada Administrative Code) 534.320. “Notice of Intent to Drill: Contents, Submission.”

NAC (Nevada Administrative Code) 534.330. “Responsibilities of Licensed Well Drillers at Drilling Site.”

NAC (Nevada Administrative Code) 534.340. “Log and Record of Work: Form; Contents.”

NRS (Nevada Revised Statutes) 534.170. “Underground Water and Wells, Well Driller to Keep Log and Records; Contents; Information to be Furnished to State Engineer; Report of Test.”

TP-7.0, Drill Site Management.

TP-8.0, Field Collection, Logging, and Processing of Borehole Samples.

TP-9.9, Measurement of Groundwater Levels Using Electric Well Sounders.

TP-11, Borehole Geophysical Logging Data Identification and Acceptance.

TABLES

Table 1

Geologic Sample Types and Numbers

| Geologic Sample Type | Core Diameter (inches) | Estimated Total Footage (feet) | Estimated Average Core Run Length (feet) | Estimated Number of Core Runs | Core Segment Length (feet) | Number of Segments per Core Run | Total Number of Segments or Samples |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Drive Core | 4 | 10 | 2.27 | 5 | 0.25 | 2 | 10 |
|  |  |  |  |  | 0.5 | 3 | 15 |
|  |  |  |  |  | 0.27a | 1 | 5 |
| Sonic Core | 6 | 145 | 5b | 29 | 2.0 | 2.5 | 72.5 |
|  | 3.5 | 145 | 5b | 29 | 2.0 | 2.5 | 72.5 |
| Grab Sample | NAc | 290 | NA | NA | NA | NA | 145d |

a  Drive core shoe sample not contained in a brass liner.

b  Conservatively assumes an average core run length of 5 feet. Target core run length is 10 feet.

c  Not applicable.

d Assumes the average layer length is 2 feet.

Table 2

Distribution of Geologic Samples

| Geologic Sample Type | Segment Length/Diameter | Total Number Of Segments/ Samples | Number of Segments/Samples Sent |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NWRPO |  |  | DOE |
|  |  |  | YMP SMFa | Laboratory | Field | YMP SMF |
| Drive Core | 0.25 feet long | 10 | 5 | 0 | 0 | 5 |
|  | 0.5 foot long | 15 | 5 | 10 | 0 | 0 |
|  | 0.27 foot long | 5 | 0 | 0 | 0 | 5 |
| Sonic Core | 6-inch diameter | 72.5 | 0 | 0 | 0 | 72.5 |
|  | 3.5-inch diameter | 72.5 | 0 | 0 | 0 | 72.5 |
| Grab Sample | NAb | 435c | 145 | 145 | 145 | 0 |

a Yucca Mountain Project Sample Management Facility.

b Not applicable.

c Each of the 145 textural layer samples is split into three subsamples.

Table 3

Summary of Laboratory Tests on Geologic Samples

| Geologic Sample Type | Number of Samples Tested |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Hydraulic Conductivity | Grain Density | Dry Bulk Density | Saturated Volumetric Water Content | Wet Sieve Particle Size Distribution | Hydrometer Particle Size Distribution |
| Drive Core | 10 | 10 | 10 | 10 | 10 | 10 |
| Sonic Core | TBDa | TBD | TBD | TBD | TBD | TBD |
| Textural Layer Grab Sample | NAb | 50 | NA | NA | 145 | 50 |

a To be determined

b Not applicable

Table 4

Laboratory Test Methods for Geologic Samples

| Laboratory Test | Method |
| --- | --- |
| Saturated Hydraulic Conductivity (Constant Head Method) | Klute, A., and C. Dirksen, 1986. Hydraulic Conductivity and Diffusivity: Laboratory Methods. In: Klute, A. (ed), Methods of Soil Analysis, Part 1, Physical and Mineralogical Methods (2nd ed.), American Society of Agronomy, Chapter 28, p. 694-700. |
| Hydrometer Analysis (Silt/Clay Break Starting with No. 4 Sieve) | ASTM D422. Standard method for Particle Size Analysis of Soils. In: 1996 Annual Book of ASTM Standards, Vol. 04.08, American Society for Testing and Materials. |
| Wet Sieve Analysis | ASTM D1140 (97). Standard method for the amount of material in soils finer than the No. 200 (75um) sieve. In: 1998 Annual Book of ASTM Standards, Vol. 04.08, American Society for Testing and Materials. |
| Volumetric Water Content | ASTM D2216-92. Method for laboratory determination of water (moisture content) of soil, rock, and soil-aggregate mixtures. In: 1996 Annual Book of ASTM Standards, Vol. 04.08, American Society for Testing and Materials. |
| Dry Bulk Density | Blake, G.R. and K.H. Hartge. 1986. Bulk Density. In: Klute, A. (ed), Methods of Soil Analysis, Part 1, Physical and Mineralogical Methods (2nd ed.), American Society of Agronomy, Chapter 13, p. 363-367. |
| Specific Gravity (Grain Density) | ASTM D854-92. Standard test method for specific gravity of soils. In: 1996 Annual Book of ASTM Standards, Vol. 04.08, American Society for Testing and Materials. |

FIGURES

ATTACHMENT A

SCOPE OF WORK FOR

DRILLING, SONIC CORING, AND CONSTRUCTION OF

ONE EWDP MONITORING WELL

ATTACHMENT B

EWDP DRILLING AND WELL CONSTRUCTION

HEALTH AND SAFETY PLAN