EWDP

DRILLING AND WELL CONSTRUCTION HEALTH AND SAFETY PLAN

It is the responsibility of the drilling contractor (Contractor) to be aware of, and comply with, the conditions of this health and safety plan.  The Contractor and any subcontractors will conduct all operations in accordance with all local, state, and federal regulations or requirements currently in effect concerning employee health and safety.  In the event that any of these regulations or requirements requires variance from the provisions set forth in this work plan, the regulatory requirements shall take precedence.

1.0	Traffic Control

Because of the remote locations of the planned wells, traffic control will not be required as part of the EWDP. However, the Contractor is required to ensure that all drivers operate their vehicles and equipment in a safe manner.  The Contractor will comply with all applicable state and local limits and restrictions and with any Nevada Department of Transportation and/or U.S. Department of Transportation requirements.  The Nye County Sheriff’s Office patrols the Nevada Test Site and all applicable county and state limits, and restrictions are enforced.

2.0	Safety Supervisor

The Contractor will appoint a safety supervisor for each crew.  This supervisor will be given the responsibility of providing a safe work environment and the authority to enforce safety as a first priority. The safety supervisor together with the NWRPO field safety officer will provide a tailgate review of work site hazards, including management of any potentially hazardous materials; trip, slip, and fall hazards; and discussion of desert environment related hazards (e.g., heat stroke and stress, dehydration, poisonous snakes and spiders) before the start of each work shift. The NWRPO field safety officer will document the tailgate safety meeting in the scientific notebook for the designated well. The safety supervisor will ensure that all equipment operators have adequate training and will inspect and test all safety equipment and devices are functioning properly including gauges, warning lights, and horns.  On a daily basis, the safety supervisor will inspect the drilling equipment daily for damage, loose parts, missing guards, fluid leaks, damaged hoses, etc.

3.0	Personnel Protective Equipment

Clothing for all onsite personnel must be appropriate for drilling and sampling operations.  Safety headgear and safety boots are required. Gloves are required for equipment handling and operation.  All onsite personnel should wear safety glasses

4.0	General Drill Site Operations

Suitable storage locations will be provided for all tools, materials, and supplies so that these items can be retrieved safely and used without injury to drill crew members, other onsite personnel, and visitors.  Tools, materials, and supplies are not to be stored on the mast.  Pipe, drill rods, casings, augers, and similar materials are to be stacked orderly on racks or sills to prevent spreading, rolling, or sliding.  Work areas, platforms, walkways, and other access ways will be kept clear of materials, debris, and obstructions.  All warning lights and lenses, controls, control linkages, and operation lights will be kept clear of mud, oil, grease, and ice.

5.0	Utility Hazards

The NWRPO will obtain utility clearances for drill sites.  In some instances, drill sites are located inside of rights-of-way for gas pipelines, telephone lines, and/or overhead electrical transmission lines.  All transmission wires and underground cables are to be considered live.   Drill rigs and other heavy equipment will maintain safe distances when used near transmission lines. The National Drilling Federation recommends a distance of at least 20 ft from any portion of the drill rig and mast to a transmission line.  This minimum distance should take into account that both the transmission line and the rig mast may be affected by high winds.

6.0	Fire Prevention

The Contractor will exercise due care at all times to ensure that fire danger is avoided.  Flammable liquids, if present, will be stored in flammable-approved containers, and will be protected from ignition sources.  Open ignition sources will be not be used in the presence of flammable liquids.  Welding or cutting will not be performed near a storage tank or container.  Gasoline or other volatile liquids will not be used as cleaning agents or around the drill rig.

7.0	Spill Prevention and Contingency Plan

Minor quantities of hazardous products and fuels may be used during the drilling process and will be properly handled by the Contractor.  All such products will be used and managed in accordance with their labeling instructions, and will be stored in a locked cabinet when not in use.  The Contractor  will inform the onsite crew of the potential hazards associated with the products that will be onsite, and spill kits will be maintained for any material kept onsite in excess of the reportable quantity. Well casings, specialized cements and grouts, foaming agents or other additives, and other routinely required drilling materials will be delivered to the site and prepared for use as needed.  Cements, grouts, and drilling additives will be mixed in a portable tank and/or a mud pit.  All excess materials will be disposed in accordance with applicable regulations.  The Contractor will be equipped with either radio or cellular telephone communication.  Spill notification information will be maintained onsite by the Contractor if reportable quantities of hazardous materials are present.

8.0	First Aid

The Contractor will have a first aid kit onsite at all times.  The safety supervisor and all NWRPO personnel should be trained in first aid methods, including CPR.

9.0	Notifications and Emergency Response

In the event of any injury or medical need, the Contractor will notify the appropriate emergency response organization.  For well sites located outside of the Nevada Test Site, the following organizations should be contacted:

Pahrump Emergency Services	911

Beatty Emergency Services	775-553-2345

Amargosa Emergency Services	775-372-5345

For well sites on the Nevada Test Site, the following organizations should be contacted:

NTS Emergency Services 	911

Ranch Control	702-295-5915

After emergency services have been contacted, the Contractor will call the NWRPO as soon as it is safe and prudent to do so.